# RMBC NUC225 — configure-networking.sh

## Engineer LAN Port + WiFi 6 AP Setup

Part of the RMBC NUC225 deployment stack:

| # | Script | Purpose |
| --- | --- | --- |
| 1 | installer-tailscale.sh | Apps, Tailscale, Nx Witness |
| 2 | prepare-cctv-storage-fixed.sh | NVMe CCTV storage mount |
| 3 | configure-networking.sh | This script — engineer access |

## What It Does

Configures two engineer access paths on the NUC without touching the production LAN (H685 router side):

| Interface | Role | IP | DHCP Pool |
| --- | --- | --- | --- |
| enp116s0 (auto-detected) | Production LAN — H685 router | 192.168.X.50 | (not managed) |
| enp115s0 (auto-detected) | Engineer LAN port | 10.50.0.1/24 | 10.50.0.50–99 |
| wlp117s0 (auto-detected) | WiFi 6 AP | 10.50.1.1/24 | 10.50.1.50–99 |

All interface names are detected dynamically — nothing is hardcoded.

WiFi SSID is derived from the unit hostname at runtime:

| Hostname | SSID |
| --- | --- |
| rmbc-1 | NX-001 |
| rmbc-15 | NX-015 |
| rmbc-194 | NX-194 |

WiFi password (fleet-wide): !#Nuc-8003#!

## Requirements

- Ubuntu Server 24.04 LTS

- Intel NUC225 with dual ethernet NICs

- RZ616 MT7922 WiFi 6 M.2 card seated

- H685 router connected to first LAN port and providing default route

- Script run as root or with sudo

The script installs these packages automatically:

hostapd   dnsmasq   rfkill   iw   network-manager

No manual apt install is needed before running.

## SCP Transfer from Windows

Open Command Prompt and navigate to where you saved the scripts:

cd C:\Users\Dane\Desktop

Transfer both scripts to the NUC:

scp configure-networking.sh configure-networking.sh rmbc@192.168.100.50:~

You will be prompted for the rmbc user password.

Important — Windows line endings: Files saved on Windows contain CRLF line endings which break bash read prompts. Always run this fix on the server immediately after SCP:

sed -i 's/\r//' cleanup-networking.sh configure-networking.sh

## File Permissions

After SCP, set execute permissions on the server:

chmod +x cleanup-networking.sh configure-networking.sh

Or run directly with bash (no chmod needed):

sudo bash configure-networking.sh

## Usage

### Fresh install (no previous networking config)

echo "YES" | sudo bash configure-networking.sh

Or with the flag:

sudo bash configure-networking.sh --yes

### Re-deploy / clean existing config first

Always run cleanup before re-running configure on a unit that has had a previous version applied:

echo "YES" | sudo bash cleanup-networking.sh

echo "YES" | sudo bash configure-networking.sh

### Interactive mode (type YES manually)

sudo bash cleanup-networking.sh

# prompted: Type YES to continue

sudo bash configure-networking.sh

# prompted: Type YES to continue

Why echo "YES" | ? The read prompt in bash can misbehave over certain SSH sessions and with Windows-transferred files. Piping YES is the reliable non-interactive method for fleet deployment.

## Verification

The script runs a self-check at the end and prints a summary. You can also verify manually:

# Check all interface IPs

ip -br addr

# Check services

systemctl status hostapd --no-pager

systemctl status dnsmasq --no-pager

systemctl status wifi-ap-ip --no-pager

# Check UFW rules

sudo ufw status numbered

### Test engineer LAN port (laptop plugged into second ethernet)

Set laptop to automatic IP, plug in, then on Windows:

ipconfig /all

Expected: IP 10.50.0.50–99, gateway 10.50.0.1

ping 10.50.0.1

ssh rmbc@10.50.0.1

### Test WiFi AP

Connect to SSID NX-XXX with password !#Nuc-8003#! then:

ipconfig /all

Expected: IP 10.50.1.50–99, gateway 10.50.1.1

ping 10.50.1.1

## Debug Commands

# dnsmasq not starting

sudo journalctl -u dnsmasq -n 30 --no-pager

# hostapd not starting

sudo journalctl -u hostapd -n 30 --no-pager

# WiFi AP IP not assigned

sudo journalctl -u wifi-ap-ip -n 20 --no-pager

# Check what is holding port 53 (should only be systemd-resolved on 127.x)

sudo ss -lntup | grep ':53\|:67'

# Check rfkill state

rfkill list all

# Check WiFi interface mode

iw dev

## Known Issues and Fixes Applied

The following issues were discovered during testing and are fixed in the current version of the script.

### Windows CRLF line endings break read prompt

Symptom: Script always shows [WARN] Aborted even when typing YES. Fix: Run sed -i 's/\r//' scriptname.sh after every SCP from Windows. The --yes flag avoids this entirely for fleet deployments.

### dnsmasq fails: failed to create listening socket for port 53

Cause: systemd-resolved holds port 53 on localhost. dnsmasq tried to bind DNS on the same port. Fix: port=0 in dnsmasq config disables DNS entirely. This unit uses systemd-resolved for DNS. dnsmasq provides DHCP only.

### dnsmasq fails: unknown interface enp115s0

Cause: dnsmasq started before the kernel had fully registered the engineer ethernet interface. Fix: Systemd drop-in in /etc/systemd/system/dnsmasq.service.d/ with After=sys-subsystem-net-devices-enp115s0.device and BindsTo= directive.

### Netplan WiFi block error: No access points defined

Cause: Using wifis: renderer block in Netplan requires access-points to be defined. This conflicts with hostapd AP mode. Fix: WiFi interface declared under ethernets: renderer instead. This tells networkd to leave the interface alone. hostapd takes full ownership.

### hostapd fails silently on first run

Cause: WiFi card was RF-blocked by kernel at boot. Fix: rfkill unblock wifi runs before hostapd starts.

### WiFi AP has no IP address after hostapd starts

Cause: hostapd puts the interface into AP mode but does not assign an IP. Fix: wifi-ap-ip.service — a oneshot systemd service that runs after hostapd and assigns 10.50.1.1/24 using ip addr replace.

### NetworkManager interferes with engineer interfaces

Cause: NetworkManager was active and could claim unconfigured interfaces. Fix: Script masks NetworkManager and NetworkManager-wait-online. Production LAN is managed by systemd-networkd via Netplan — this is correct for Ubuntu Server and does not require NetworkManager.

### UFW DHCP rule blocked fresh engineer clients

Cause: Previous script versions allowed DHCP by subnet, not by interface. On a fresh client connection the client has no IP yet and the subnet rule did not match. Fix: UFW rules applied per-interface name (ufw allow in on enp115s0).

### WiFi AP limited to 2.4GHz only

Cause: hw_mode=g in hostapd config restricts to 2.4GHz band. Fix: hw_mode=a enables 5GHz. The RZ616 MT7922 supports 5GHz/6GHz.

### packages not installed before require_cmds ran

Cause: require_cmds ran before install_packages, so iw, rfkill and nmcli were checked before being installed and the script exited. Fix: install_packages now runs first. require_cmds runs after as a final verification that all tools are present.

## Files Created by This Script

| Path | Purpose |
| --- | --- |
| /etc/netplan/60-rmbc-engineer.yaml | Static IP for engineer ethernet port |
| /etc/netplan/61-rmbc-wifi.yaml | Marks WiFi interface as unmanaged by networkd |
| /etc/hostapd/hostapd.conf | hostapd WiFi 6 AP configuration |
| /etc/default/hostapd | Sets DAEMON_CONF path for hostapd init |
| /etc/dnsmasq.d/rmbc-networking.conf | DHCP-only dnsmasq config for both interfaces |
| /etc/systemd/system/wifi-ap-ip.service | Assigns static IP to WiFi AP after hostapd starts |
| /etc/systemd/system/dnsmasq.service.d/rmbc-wait-interfaces.conf | Makes dnsmasq wait for interfaces |
| /var/log/configure-networking.log | Script run log |

## Cleaning Up

To remove all engineer networking config and return to a clean state:

echo "YES" | sudo bash cleanup-networking.sh

This removes all files listed above, flushes stale IPs from non-production interfaces, and re-applies Netplan for the production LAN only.

RMBC NUC225 CCTV Deployment — Rotherham Metropolitan Borough Council